DEPARTMENT OF PETROLEUM ENGINEERING

FINAL YEAR PROJECT REPORT

DEVELOPMENT OF A RANDOM FOREST-BASED VERTICAL LIFT PERFORMANCE MODEL FOR MULTIPHASE WELLBORE FLOW PREDICTION.

PARTIAL FUFILMENT OF THE REQUIREMENT FOR THE AWARD

OF

BACHELORS IN ENGINEERING (B.ENG)

IN

DEPARTMENT OF PETROLEUM ENGINEERING

CHAPTER ONE

INTRODUCTION

1.1 Background of the Study

The production of crude oil and natural gas from underground reservoirs involves several interconnected processes. While reservoir engineers are concerned with the ability of a reservoir to deliver fluids, production engineers focus on ensuring that these fluids can be transported efficiently from the reservoir to the surface facilities. One area that plays a critical role in this process is the Vertical Lift Performance (VLP).

Vertical Lift Performance (VLP) is a fundamental concept in production engineering that describes the pressure losses encountered as fluids flow from the bottom of the well to the surface through production tubing. VLP analysis establishes the relationship between the production rate and flowing bottom-hole pressure, making it an essential component of the nodal analysis used in well performance evaluation. Traditionally, engineers use empirical and mechanistic correlations such as those developed by Gilbert, Hagedorn and Brown, Duns and Ros, Beggs and Brill, Mukherjee and Brill, and Ansari to estimate pressure losses in multiphase flow systems. These correlations have served the petroleum industry for decades and remain widely used in commercial simulators such as Pipesim and PROSPER.

However, predicting pressure losses in multiphase vertical flows remains a challenge. Oil, gas, and water often flow simultaneously through production tubing, creating complex flow patterns, such as bubble, slug, churn, and annular flows. These flow regimes continuously change with the pressure, temperature, fluid properties, tubing geometry, and production rate. Consequently, conventional correlations frequently produce varying levels of prediction accuracy when applied to wells operating under conditions different from those used during the development of the correlations.

In Nigeria, particularly within the Niger Delta region, many producing wells have matured. Reservoir pressure depletion, increasing water production, gas breakthrough, and aging infrastructure have introduced additional challenges to the optimization of production. In such situations, accurate prediction of well performance becomes increasingly important because operational decisions, such as tubing design, artificial lift selection, gas lift optimization, and production forecasting, depend heavily on reliable VLP calculations.

In recent years, the emergence of machine learning (ML) has provided new opportunities for solving complex engineering challenges. Machine learning techniques can identify hidden patterns within large datasets without requiring explicit mathematical relationships. Unlike conventional empirical correlations developed using predefined assumptions, machine learning models learn directly from historical data and can adapt to nonlinear relationships among variables. The application of machine learning in petroleum engineering has expanded rapidly in recent years. Researchers have successfully applied machine learning to reservoir characterization, drilling optimization, production forecasting, artificial lift design, pressure prediction, flow assurance, and well-performance analysis. Recent studies have shown that machine learning models can accurately predict pressure drop, flowing bottom-hole pressure, production rates and multiphase flow behavior under complex operating conditions.

Among the various machine learning algorithms available, Random Forest has gained significant attention owing to its robustness, ability to handle nonlinear relationships, resistance to overfitting, and interpretability. The random Forest method combines multiple decision trees to generate highly accurate predictions while maintaining computational efficiency. Its effectiveness has been demonstrated in several petroleum engineering applications that involve complex production datasets.

During my review of published journal articles and technical papers, I observed that many recent studies have focused on predicting flowing bottom-hole pressure, production rates, or liquid loading conditions using machine learning techniques. However, relatively few studies have focused specifically on developing Random Forest-based Vertical Lift Performance models for multiphase wellbore flow prediction. This observation further strengthened my interest in conducting this research.

Therefore, this study focuses on the development of a new Vertical Lift Performance prediction model using the Random Forest (RF) algorithm. The intention is not necessarily to replace traditional VLP methods entirely but rather to investigate whether a data-driven approach can provide improved prediction accuracy and greater adaptability under varying production conditions.

## 1.2 Statement of the Problem

Accurate prediction of Vertical Lift Performance is important for effective well management and production optimization. Unfortunately, predicting pressure losses in multiphase flow systems remains one of the most difficult challenges in production engineering.

Traditional VLP correlations have served the industry for decades, but many were developed using experimental datasets obtained under specific operating conditions. Consequently, their applicability may be limited when field conditions differ significantly from those used during model development.

From the literature reviewed, it became clear that engineers often obtain different pressure predictions when different VLP correlations are applied to the same wells. Such inconsistencies create uncertainty in production planning and decision-making.

This issue is particularly relevant in mature oil fields, where the operating conditions change continuously over time. Increasing water cut, fluctuating gas-liquid ratios, declining reservoir pressure, and changing flow regimes can reduce the accuracy of traditional prediction methods.

Furthermore, conventional VLP calculations become computationally intensive when repeated simulations are required. In some situations, engineers may require faster and more adaptable prediction tools capable of handling large amounts of production data.

Despite recent advances in machine learning, relatively limited work has been done on developing Random Forest-based models specifically for Vertical Lift Performance prediction. This creates a research gap that this study aims to address.

1.3 Aim of the Study

The aim of this study was to develop a Random Forest-based Vertical Lift Performance model for predicting multiphase wellbore flow behavior in oil and gas production systems.

## 1.4 Objectives of the Study

The specific objectives were as follows:

- To study the principles of Vertical Lift Performance in multiphase flow systems.

- To review existing VLP prediction methods used in the petroleum industry.

- To identify the key parameters affecting pressure losses in vertical wellbores.

- A suitable dataset was developed for model development.

- To train and validate a Random Forest model for VLP prediction.

- The performance of the developed model was evaluated using statistical indicators.

- To compare the developed model with traditional prediction approaches.

- The practical applicability of the model in petroleum production operations was assessed.

1.5 Research Questions

This study seeks to answer the following questions:

- What are the major factors influencing Vertical Lift Performance in multiphase wellbore flow systems?

- How accurate are the traditional VLP correlations under varying production conditions?

- Can Random Forest algorithms accurately predict Vertical Lift Performance?

- How does the Random Forest model compare with conventional VLP prediction methods?

- What advantages can machine learning provide for well-performance prediction and production optimization?

1.6 Significance of the Study

This study is important from both academic and industrial perspectives.

As a Petroleum Engineering student, I have observed that many production engineering calculations still rely heavily on empirical correlations developed several decades ago. Although these correlations remain valuable, advances in computing and data science provide opportunities to explore alternative approaches.

Academically, this study contributes to the growing body of knowledge on machine learning applications in petroleum engineering. This demonstrates how data-driven methods can be integrated with conventional engineering concepts to solve practical field problems.

For production engineers, the findings may provide insights into the potential use of Random Forest models for rapid well performance prediction.

For oil and gas companies operating in Nigeria and similar producing regions, improved VLP prediction can contribute to better production optimization, reduced uncertainty, and more efficient resource utilization.

Furthermore, this study serves as a foundation for future research involving advanced machine learning techniques, such as XGBoost, deep learning, and hybrid physics-informed machine learning models.

1.8 Scope of the Study

This study focuses on the development of a Random Forest-based model for predicting Vertical Lift Performance in multiphase wellbore flow systems.

The study considered production parameters, including:

- Flow rate

- Tubing diameter

- Gas-liquid ratio

- Water cut

- Pressure

- Temperature

- Fluid properties

The model development process included data collection, preprocessing, model training, validation, testing, and performance evaluation.

This study was limited to supervised machine learning techniques, with Random Forest serving as the primary predictive algorithm.

1.9 Limitations of the Study

Several limitations were encountered during this research.

The first challenge is obtaining high-quality field production datasets. Access to real industry data is often restricted because of confidentiality agreements between companies. Another limitation is the computational resources. Although Random Forest is relatively efficient, model optimization still requires significant computational effort when large datasets are used.

This study was also limited by the quality and representativeness of the available data. Because machine learning models learn from historical observations, the prediction accuracy depends heavily on the dataset used for training. Finally, the developed model may require retraining before being applied to fields with significantly different operating conditions.

1.10 Organization of the Report

This project report is organized into five chapters as follows.

Chapter One introduces the study and presents the problem statement, objectives, significance, scope, limitations, and research questions. Chapter Two reviews the relevant literature on Vertical Lift Performance, traditional prediction methods, machine learning applications, and previous studies related to the research topic. Chapter Three presents the methodology adopted for data collection, preprocessing, Random Forest model development, and model evaluation. Chapter Four presents the results, analysis, discussion, and performance comparisons. Chapter Five summarizes the findings, presents the conclusions, and provides recommendations for future work.

CHAPTER TWO

LITERATURE REVIEW

2.1 Introduction

This chapter presents a review of the literature related to Vertical Lift Performance (VLP), traditional methods used for VLP prediction, machine learning applications in petroleum engineering, and recent studies related to multiphase flow prediction. The review was conducted using journal articles, conference papers, textbooks, and technical publications published mainly between 2020 and 2025.

During the literature review process, I found that the petroleum industry is increasingly embracing artificial intelligence and machine learning (ML) technologies. Many researchers have explored data-driven methods as alternatives to conventional engineering correlations. However, the integration of machine learning into Vertical Lift Performance prediction remains an evolving research area. This observation was one of the motivations for this study.

The purpose of this chapter is to summarize previous studies and critically examine their findings, identify their strengths and weaknesses, and establish the research gap that this project intends to address.

2.2 Concept of Vertical Lift Performance

Vertical Lift Performance refers to the pressure required to transport reservoir fluids from the bottom of a well to the surface through the production tubing. It forms the outflow component of the nodal analysis and plays a major role in determining the production capacity of a well.

During my study of production engineering, I came to understand that VLP is more than just a pressure calculation. It influences almost every production-related decision, including tubing design, artificial lift selection, gas lift optimization, production forecasting, and well-performance analysis.

As fluids travel upward through the tubing, pressure losses occur owing to three primary mechanisms:

2.2.1 Hydrostatic Pressure Loss

Hydrostatic pressure loss occurs because of the weight of the fluid column in the tubing. The denser the fluid, the greater the pressure required to lift it to the surface of the fluid.

2.2.2 Frictional Pressure Loss

Frictional losses result from the interaction between flowing fluids and tubing walls. These losses increase with the production rate and can become significant in high-rate wells.

2.2.3 Acceleration Pressure Loss

Acceleration losses occur when the fluid velocity changes along the wellbore. Although they are usually smaller than hydrostatic and frictional losses, they become important under certain flow conditions.

An interesting observation from the literature is that hydrostatic losses often dominate in low-rate wells, whereas frictional losses become increasingly important at higher production rates. This implies that the relative importance of each pressure loss component can change significantly throughout the production life of a well.

2.3 Multiphase Flow and Its Influence on Vertical Lift Performance

One reason why VLP prediction is difficult is that petroleum wells rarely produce a single fluid phase. In most producing wells, oil, gas, and water flow simultaneously.

Unlike single-phase flow systems, where fluid behavior is relatively predictable, multiphase flow introduces additional complexities because different flow patterns can develop within the tubing.

Common flow regimes identified in the literature include

Bubble Flow

Gas bubbles are dispersed throughout the continuous liquid phase.

Slug Flow

Large gas pockets alternated with liquid slugs.

Churn Flow

The flow became highly unstable and oscillatory.

Annular Flow

The gas occupies the central region, whereas the liquid flows along the tubing walls.

While reviewing production engineering textbooks, I noticed that predicting transitions between these flow regimes remains one of the greatest challenges in VLP modelling. Even small changes in the gas-liquid ratio or flow rate can cause flow regime transitions, leading to significant changes in pressure losses.

This explains why no single VLP correlation performs perfectly under all the operating conditions.

2.4 Traditional Vertical Lift Performance Prediction Methods

Before the emergence of machine learning, petroleum engineers relied almost entirely on empirical and mechanistic models for VLP predictions. These models remain widely used today because they are embedded in commercial software such as Pipesim and Prosper.

However, after reviewing multiple studies, I found that each model has its strengths and weaknesses.

2.4.1 Gilbert Correlation

The Gilbert correlation was one of the earliest methods developed for flowing wells, particularly gas-lift ones.

Advantages

- Easy to implement.

- It requires relatively few input parameters.

- Computationally simple.

Limitations

- Developed using limited datasets.

- The accuracy decreases outside the calibration conditions.

- However, it is not suitable for many modern production systems.

Although Gilbert's work was groundbreaking during its time, several recent studies have suggested that more advanced methods generally provide better prediction accuracy.

2.4.2 Hagedorn and Brown Correlation

This correlation remains one of the most widely used methods for vertical multiphase flow calculation.

Advantages

- Extensively validated.

- Widely accepted in the industry.

- It performs reasonably well under several operating conditions.

Limitations

- Can underperform under high gas fractions.

- It may produce errors for wells operating outside the original experimental range.

Many of the papers reviewed still use Hagedorn and Brown as a benchmark when evaluating newer prediction models.

2.4.3 Duns and Ros Correlation

Duns and Ros introduced flow regime considerations into the pressure drop calculations.

Advantages

- Better representation of multiphase flow behavior.

- Improved handling of flow regime transitions is required .

Limitations

- This implementation is more complicated.

- Sensitive to flow pattern identification.

While reading different studies, I found that engineers often obtain different results depending on how the flow regime transitions are handled.

2.4.4 Beggs and Brill Correlation

The Beggs and Brill correlation remains one of the most commonly applied correlations because of its versatility.

Advantages

- This is applicable to both vertical and inclined wells.

- It is widely available in commercial simulators.

Limitations

- It assumes idealized flow patterns.

- The accuracy varies depending on the operating conditions.

Several studies have reported that Beggs and Brill perform well in some wells but poorly in others, emphasizing the need for careful correlation selection.

2.5 Environmental and Operational Implications of Poor VLP Prediction

One aspect that has received less attention in some publications is the broader consequence of inaccurate VLP predictions.

If the pressure losses are underestimated, the production forecasts may become overly optimistic. Conversely, overestimating the pressure losses can lead to conservative operating decisions and reduced production opportunities.

Poor VLP predictions may result in the following:

- Inefficient gas-lift operations.

- Excessive energy consumption is another issue.

- Improper tubing design.

- Increased operating expenses were also observed.

- This leads to reduced recovery efficiency.

In mature fields, such as those found in the Niger Delta, these issues can significantly affect economic performance because operators are already working with declining reservoir pressures and increasing water production.

2.6 Machine Learning in Petroleum Engineering

The rapid growth in data availability in the petroleum industry has encouraged the adoption of machine learning techniques.

Machine learning differs from traditional engineering models in that it does not require explicit mathematical equations to describe the system. Instead, it learns the relationships directly from the historical data.

Initially, I was uncertain about the effective application of machine learning to petroleum engineering challenges. because several engineering processes are governed by physical laws. However, after reviewing recent studies, I realized that machine learning does not necessarily replace physics-based methods. Rather, it provides additional tools for handling highly complex systems where conventional models struggle.

Machine learning applications in petroleum engineering now include the following:

- Reservoir Characterization.

- Production forecasting.

- Well test interpretation.

- Drilling optimization.

- Equipment failure prediction.

- Artificial-lift optimization.

- Pressure-loss prediction.

This increasing adoption suggests that machine learning is becoming an important component of modern petroleum engineering practices.

2.7 Random Forest Algorithm

Among the various machine learning techniques available, Random Forest was selected for this study.

Random Forest is an ensemble learning algorithm that combines multiple decision trees to improve prediction performance.

One reason for selecting Random Forest is that many researchers have described it as one of the most reliable algorithms for engineering datasets. Unlike some advanced deep learning models that require large amounts of data and extensive computational resources, Random Forest often performs well even with moderate-sized datasets.

Advantages of Random Forest

- It effectively handles nonlinear relationships.

- It is resistant to overfitting.

- Can process large numbers of variables.

- It provides feature importance rankings.

- It requires relatively little parameter tuning.

Limitations

- Less interpretable than simple regression models.

- The performance depends on the data quality.

- Large forests may increase the computational requirements.

Despite these limitations, most studies reviewed reported strong predictive performance when Random Forest was applied to petroleum engineering datasets.

2.8 Review of Previous Studies (2020–2025)

The major objective of this review was to examine recent developments in machine learning applications related to VLP and well-performance prediction.

During the review process, we noticed that many studies focused on pressure prediction, production forecasting, and artificial lift optimization rather than direct VLP modelling.

## 2.8.1 Barjouei and Yang (2021)

Barjouei and Yang (2021) investigated the application of machine learning techniques to predict liquid production rates in oil wells. The authors compared Support Vector Regression (SVR), Decision Trees (DT), Random Forest (RF), Artificial Neural Networks (ANN), and Deep Learning models using field production data. Their results showed that machine learning models generally outperformed traditional empirical prediction approaches, with Random Forest demonstrating strong predictive capability while requiring relatively lower computational effort. This study highlighted the ability of Random Forest to capture nonlinear relationships among production variables. However, this research focused primarily on production rate prediction rather than vertical lift performance prediction. (Springer)

### Relevance to This Study

This study demonstrates the suitability of Random Forest for petroleum engineering datasets and supports its selection as the primary algorithm in the present research.

## 2.8.2 Wang et al. (2021)

Wang et al. developed machine learning models to predict daily oil production using downhole pressure and temperature measurements. The authors utilized Long Short-Term Memory (LSTM) neural networks and reported significant improvements in forecasting accuracy compared with conventional methods. Their study demonstrated the growing potential of data-driven approaches in production engineering applications. (Lume)

### Observation

Although this study focused on production forecasting, it showed that operational well data contain sufficient information for developing predictive petroleum engineering models.

## 2.8.3 Ibrahim et al. (2022)

Ibrahim et al. compared multiple machine learning algorithms for hydrocarbon production forecasting. Their evaluation included Random Forest, Gradient Boosting, and Neural Networks. The researchers concluded that Random Forest provided a strong balance between prediction accuracy, model stability, and computational efficiency.

### Observation

One aspect I noticed while reviewing this type of study is that the Random Forest repeatedly appears among the best-performing algorithms, even when different datasets are used.

## 2.8.4 Ahmed et al. (2023)

Ahmed et al. developed machine learning models for pressure-drop estimation in multiphase flow systems. The authors demonstrated that machine learning algorithms can effectively learn the relationship between fluid properties, flow conditions, and pressure losses. Their findings suggest that data-driven approaches can reduce the prediction errors commonly associated with traditional correlations. (ResearchGate)

### Relevance to This Study

Pressure-drop prediction is one of the key components of Vertical Lift Performance modelling. Therefore, the findings provide important support for the use of machine learning in VLP prediction.

## 2.8.5 Sun et al. (2023)

Sun and colleagues investigated the prediction of oil-water flow patterns using both Random Forest and neural network models. Using multiphase flow experimental data, the authors demonstrated that Random Forest could successfully classify flow patterns under varying operating conditions. (MDPI)

### Observation

Flow-pattern prediction is directly linked to VLP analysis because pressure losses depend strongly on the prevailing flow regime. The study suggests that machine learning can successfully capture complex multiphase flow behavior.

## 2.8.6 Quintino et al. (2024)

Quintino and co-workers proposed a machine-learning framework for gas-liquid pipe flow prediction. The researchers applied deep ensemble learning models and tested their performance using laboratory multiphase flow data. Results indicated strong predictive accuracy and good generalization capability. (OnePetro)

### Limitation

The study focused on pipeline systems rather than vertical wellbores.

## 2.8.7 Bolourchifard et al. (2024)

Bolourchifard and co-authors developed machine-learning pipelines for pressure-drop estimation in two-phase adiabatic flow systems. Their study employed experimentally generated datasets and demonstrated that machine-learning methods could predict pressure losses with high accuracy. (MDPI)

### Relevance

This work further strengthens the argument that machine learning techniques can effectively model pressure-loss behavior in complex flow systems.

## 2.8.8 Nemer et al. (2024)

Nemer and colleagues compared Decision Tree Regression, Random Forest Regression, and XGBoost models for oil and gas production forecasting. Their findings showed that Random Forest achieved the highest prediction accuracy among the tested models. (jqcsm.qu.edu.iq)

### Observation

One recurring pattern observed across several studies is the consistent performance of Random Forest in petroleum engineering applications.

## 2.8.9 SPE Production Forecasting Study (2024)

A study published through the Society of Petroleum Engineers (SPE) presented a novel machine-learning approach for production forecasting using historical production data and well completion parameters. The authors reported improved forecasting capability compared with conventional techniques. (JPT)

### Importance

The study illustrates the growing acceptance of machine learning technologies by industry practitioners.

## 2.8.10 Akbari et al. (2025)

Akbari and colleagues investigated machine learning-based prediction of well performance under multiphase flow conditions. The study compared machine learning approaches with analytical and empirical methods traditionally used in petroleum engineering. The authors concluded that machine learning models provided superior adaptability under varying operating conditions. (Nature)

### Relevance

This study is particularly relevant because it directly addresses multiphase flow prediction, which forms the basis of Vertical Lift Performance analysis.

## 2.8.11 Alakbari et al. (2025)

Alakbari and co-authors developed an Adaptive Neuro-Fuzzy Inference System (ANFIS) model for predicting pressure drops in vertical multiphase wells. Using 335 experimental records, the model demonstrated significant improvements over traditional empirical correlations. (Nature)

### Observation

This study reinforces one of the major arguments of the present research: traditional VLP correlations may not always provide sufficient prediction accuracy under complex flow conditions.

## 2.8.12 Energy & Fuels Study on Gas-Lift Wells (2025)

Researchers developed physics-based and data-driven machine learning models for predicting flowing bottom-hole pressure in gas-lifted wells. The proposed models integrated operational, reservoir, and production parameters and achieved strong prediction performance. (American Chemical Society Publications)

### Relevance

Flowing bottom-hole pressure is a critical component of VLP calculations, making this study highly relevant to the present work.

## 2.8.13 Jiang et al. (2025)

Jiang and colleagues proposed a meta-learning enhanced Random Forest ensemble for oil production forecasting. Their results demonstrated improved forecasting accuracy compared with conventional Random Forest implementations. (MDPI)

### Observation

This study shows that Random Forest continues to evolve and remains an active area of machine learning research.

## 2.8.14 Uribe-Tarazona et al. (2025)

Uribe-Tarazona and co-authors developed machine learning models for identifying oil-water two-phase flow patterns. The study utilized 1,846 experimental data points and achieved successful classification of multiple flow regimes. (Frontiers)

### Relevance

Flow-pattern identification is directly related to pressure-loss prediction and VLP modelling.

## 2.8.15 Zeinula et al. (2025)

Zeinula and colleagues proposed a hybrid production forecasting framework combining Random Forest and Long Short-Term Memory models. Their findings indicated that hybrid machine-learning approaches could improve prediction reliability in complex petroleum systems. (Nazarbayev University)

### Limitation

The research focused on production forecasting rather than direct VLP prediction.

# 2.9 Critical Analysis of Previous Studies

After reviewing the literature, several important observations became clear.

First, there is a noticeable shift within petroleum engineering from purely empirical correlations toward data-driven prediction methods. Studies published between 2020 and 2025 consistently demonstrate that machine learning techniques can achieve prediction accuracies comparable to or better than conventional engineering correlations.

Second, Random Forest appears repeatedly among the highest-performing algorithms. This may be due to its ability to handle nonlinear relationships, noisy data, and interactions among multiple variables without requiring extensive parameter tuning.

Third, despite the growing number of machine learning studies, relatively few researchers have focused specifically on Vertical Lift Performance prediction. Most published works concentrate on:

- Production forecasting

- Pressure-drop prediction

- Flow-pattern classification

- Flowing bottom-hole pressure prediction

- Artificial lift optimization

Finally, very limited literature was found that specifically considers production conditions representative of Niger Delta oil fields. Most available studies use datasets from North America, the Middle East, Europe, or laboratory experiments.

# 2.10 Identified Research Gap

Based on the reviewed literature, the following gaps exist:

- Limited studies focus specifically on Random Forest-based Vertical Lift Performance modelling.

- Most machine learning studies emphasize production forecasting rather than wellbore pressure-loss prediction.

- Few studies compare Random Forest predictions directly with conventional VLP correlations used in production engineering practice.

- Limited research addresses conditions representative of Niger Delta production systems.

- There is a need for simple and interpretable machine learning models that petroleum engineers can easily implement in field operations.

The present study seeks to address these gaps by developing and evaluating a Random Forest-based Vertical Lift Performance model for multiphase wellbore flow prediction.

2.11 Summary of Literature Review

The literature reviewed shows that Vertical Lift Performance remains an essential component of petroleum production engineering. Traditional correlations continue to provide useful engineering solutions, but their prediction accuracy may vary under complex multiphase flow conditions.

Recent developments in machine learning have created opportunities for improved prediction of well performance parameters. Among the available algorithms, Random Forest appears particularly suitable because of its ability to model nonlinear relationships while maintaining robustness and ease of implementation.

The review also revealed that although significant progress has been made in applying machine learning to petroleum engineering problems, there remains a need for dedicated Random Forest-based Vertical Lift Performance models. This study therefore seeks to contribute to this growing area of research by developing and evaluating a new Random Forest-based VLP prediction model for multiphase wellbore flow systems.